ADC with stm32 with multiple channels

# ADC with dma

## Active clock (RCC configurations)

## Clock configurations

## ADC1 parameter settings

-Activate IN1 and IN2.

-Clock Prescaler : The ADC work at 36 MHZ as maximum frequency , the clock line of adc id ---APB2 with frequency 84 MHZ, so we divide PCLK2 by 4.

-Scan conversion Mode : Enabled for multi ADC reading.

-Continuos conversion mode: Enabled.

-DMA Continuos request: Enabled.

-Number of conversion: 2 because we use 2 channels.

-Press Rank and choose channels required.

Note: ADC is so fast, so that case fast DMA interrupt and the code wouldn’t work so you have to make it slowest, so increase the simple time and decrease clock prescaler.

## ADC DMA settings

-Press add. Select ADC1. You can choose stream 0. Choose priority u want , we will use very high. Choose circular mode .

-Increment address is choosen if address change, address not change in peripheral but change at memory so we will choose only memory.

-Data Width, Byte is 8, half word 16 and word is 32 bit, because ADC is 12 bit , we can choose Half word it is okay for us.

Now generate the code, and let’s start writing code.

## Code writing

Lets define three variables in our code. The first is an array it’s the dma buffer, to the second and the third varibles we will write the data form the buffer.

/* USER CODE BEGIN PV */ uint16_t adc_value[2]; uint16_t adc1_ch0,adc1_ch1; /* USER CODE END PV */

Refer to stm32f4xx_hal_adc.c we can learn how to read adc values with dma.

*** DMA mode IO operation *** ============================== [..] (+) Start the ADC peripheral using HAL_ADC_Start_DMA(), at this stage the user specify the length of data to be transferred at each end of conversion (+) At The end of data transfer by HAL_ADC_ConvCpltCallback() function is executed and user can add his own code by customization of function pointer HAL_ADC_ConvCpltCallback (+) In case of transfer Error, HAL_ADC_ErrorCallback() function is executed and user can add his own code by customization of function pointer HAL_ADC_ErrorCallback (+) Stop the ADC peripheral using HAL_ADC_Stop_DMA()

-Start ADC DMA after init, don’t forget to cast buffer to 32 bit .

/* USER CODE BEGIN 2 */ HAL_ADC_Start_DMA(&hadc1, (uint32_t*)adc_value, 2); /* USER CODE END 2 */

- In while loop let’s write data from the adc_value buffer, to the variables.

while (1) { /* USER CODE END WHILE */ /* USER CODE BEGIN 3 */ adc1_ch0 = adc_value[0]; adc1_ch1= adc_value[0]; }